# Safe, Affordable, Effective Retrofits

# Data Management Plan

May 5, 2017 (update date)

Table of Contents

Context	2

Title: Safe, Affordable, Effective Retrofits (SEAR)	2

Data Collection	2

Policies	2

Responsibilities	2

Organization of data	2

Documentation and Metadata	3

Data Documentation	3

Metadata	3

Storage and Security	3

Preservation and Sharing	3

Appendix	4

Data Measurement and Recording Standards	4

# Context

## Title: Safe, Affordable, Effective Retrofits (SEAR)

Purpose: This effort focuses on testing retrofit wall assemblies with vapor open exterior insulation. The potential wall configurations chosen are based on the findings from the initial project work that included hygrothermal modeling and contractor feedback on cost of construction (see, http://www.cchrc.org/safe-effective-affordable-retrofits).

Funders: Alaska Housing Finance Corporation

Duration: 18 months

Folder Locations:  \\cabin\CCHRC\Building Science Research\Projects\Safe, Effective, Affordable Retrofits (SEAR) 2017-250

Proposal Documents: \\cabin\CCHRC\Building Science Research\Projects\Safe, Effective, Affordable Retrofits (SEAR) 2017-250\New Proposal for SEAR

## Data Collection

In order to meet the objectives digital data will be collected, stored, and analyzed. Data will be collected via CSI CR100 dataloggers and stored on the CCHRC data storage site, http://cchrc.arsc.edu/

## Policies

The loggers will scan the sensors on 5 second intervals. The data will be averaged and stored in 60 minute intervals, as well as daily intervals.

## Responsibilities

The project manager is responsible for making certain this data is being collected and is in an error free format.

CCHRC IT department is responsible for providing a consistent network connection for the dataloggers.

The project technician will be responsible for keeping constant humidity in the MTL over the course of the study.

# Organization of data

#### File structure

The CSI loggers output .csv files organized by date. The .csv files can be exported to Excel for analysis but the original .csv files will be retained on http://cchrc.arsc.edu/, always.

#### Naming conventions

The sensors get labels when they are deployed. Each name will start with the with the Wall#_  and end with _units. All walls have specific locations for sensors these are denoted in the name using ‘U’ for upper, ‘M’ for middle, and ‘B’ for bottom. Sensors are also located in 3 locations across the middle of the wall, ‘O’ for outer, ‘I’ for inner, and ‘T’ for outer Tyvek. Figure 1 is a draft of sensor locations, better graphics are in the Appendix. Table 1 lists all the sensor names for wall specific sensors

| Wall1_MO_RH | Wall1_MO_RH_C | Wall1_MO_C | Wall1_U_MC | Wall1_U_MC_C |
| --- | --- | --- | --- | --- |
| Wall1_UI_RH | Wall1_UI_RH_C | Wall1_HFS_C | Wall1_M_MC | Wall1_M_MC_C |
| Wall1_MI_RH | Wall1_MI_RH_C | Wall2_MO_C | Wall1_B_MC | Wall1_B_MC_C |
| ROOM_RH1 | ROOM_RH1_RH_C | Wall2_HFS_C | Wall2_U_MC | Wall2_U_MC_C |
| Wall2_MO_RH | Wall2_MO_RH_C | Wall3_MO_C | Wall2_M_MC | Wall2_M_MC_C |
| Wall2_UI_RH | Wall2_UI_RH_C | Wall3_HFS_C | Wall2_B_MC | Wall2_B_MC_C |
| Wall2_MI_RH | Wall2_MI_RH_C | Wall4_MO_C | Wall3_U_MC | Wall3_U_MC_C |
| Wall3_MO_RH | Wall3_MO_RH_C | Wall4_HFS_C | Wall3_M_MC | Wall3_M_MC_C |
| Wall3_UI_RH | Wall3_UI_RH_C | Wall5_MO_C | Wall3_B_MC | Wall3_B_MC_C |
| Wall3_MI_RH | Wall3_MI_RH_C | Wall5_HFS_C | Wall4_U_MC | Wall4_U_MC_C |
| ROOM_RH2 | ROOM_RH2_RH_C | Wall6_MO_C | Wall4_M_MC | Wall4_M_MC_C |
| Wall4_MO_RH | Wall4_MO_RH_C | Wall6_HFS_C | Wall4_B_MC | Wall4_B_MC_C |
| Wall4_UI_RH | Wall4_UI_RH_C | Wall7_MO_C | Wall5_U_MC | Wall5_U_MC_C |
| Wall4_MI_RH | Wall4_MI_RH_C | Wall7_HFS_C | Wall5_M_MC | Wall5_M_MC_C |
| Wall5_MO_RH | Wall5_MO_RH_C | Wall8_MO_C | Wall5_B_MC | Wall5_B_MC_C |
| Wall5_UI_RH | Wall5_UI_RH_C | Wall8_HFS_C | Wall6_U_MC | Wall6_U_MC_C |
| Wall5_MT_RH | Wall5_MT_RH_C | Wall9_MO_C | Wall6_M_MC | Wall6_M_MC_C |
| Wall5_MI_RH | Wall5_MI_RH_C | Wall9_HFS_C | Wall6_B_MC | Wall6_B_MC_C |
| Wall6_MO_RH | Wall6_MO_RH_C |  | Wall7_U_MC | Wall7_U_MC_C |
| Wall6_UI_RH | Wall6_UI_RH_C |  | Wall7_M_MC | Wall7_M_MC_C |
| Wall6_MT_RH | Wall6_MT_RH_C |  | Wall7_B_MC | Wall7_B_MC_C |
| Wall6_MI_RH | Wall6_MI_RH_C |  | Wall8_U_MC | Wall8_U_MC_C |
| Wall7_MO_RH | Wall7_MO_RH_C |  | Wall8_M_MC | Wall8_M_MC_C |
| Wall7_UI_RH | Wall7_UI_RH_C |  | Wall8_B_MC | Wall8_B_MC_C |
| Wall7_MT_RH | Wall7_MT_RH_C |  | Wall9_U_MC | Wall9_U_MC_C |
| Wall7_MI_RH | Wall7_MI_RH_C |  | Wall9_M_MC | Wall9_M_MC_C |
| Wall8_MO_RH | Wall8_MO_RH_C |  | Wall9_B_MC | Wall9_B_MC_C |
| Wall8_UI_RH | Wall8_UI_RH_C |  |  |  |
| Wall8_MI_RH | Wall8_MI_RH_C |  |  |  |
| Wall9_MO_RH | Wall9_MO_RH_C |  |  |  |
| Wall9_UI_RH | Wall9_UI_RH_C |  |  |  |
| Wall9_MI_RH | Wall9_MI_RH_C |  |  |  |

All data files will start with SEAR_ . Wall specific data files will be called SEAR_wall1_ or wall2 etc…  There is an overall hourly and daily average data table. There is a table for each wall averaged hourly. There is also an hourly  diagnostic and raw data table.

#### Formats and Units

All numbers will be no longer than 4 values after the decimal place. The data will be stored as a .csv. SI units will be used for raw data.

#### Versioning

Raw data files will be saved in the data file within the project file. Any analyzed data will have the date of the analyzed data in its name. Data should not be overwritten if new analysis takes place, a new version of the data should be saved with a new name that includes the date.

# Documentation and Metadata

## Data Documentation

All digital data and documentation for this project will be stored here http://www.cchrc.org/research-data-system

## Metadata

Data on the sensors in this project will follow a very specific format. There is an overall data standards document outlined below.

#### Data Standards

Detailed data standards are appended to this document.  The data standards include details about each sensor, its purpose, and location.

#### Metadata Spreadsheet

Each data file will have a metadata spreadsheet. The spreadsheet will be uploaded to http://www.cchrc.org/research-data-system along with the data

# Storage and Security

Sensor data will be stored on the ARSC server for CCHRC and accessible here, http://www.cchrc.org/research-data-system, this data is backed up to tape on a regular basis.

# Preservation and Sharing

Sensor data will be stored on http://www.cchrc.org/research-data-system. Final data analysis and reporting will be available on the CCHRC website when the project is complete.

# Appendix

# Data Measurement and Recording Standards

Last Update: 05/04/2017

Last Update By: Robbin Garber-Slaght

Weather Data

The existing MET station on the RTF roof will be used. This data is also housed on http://cchrc.arsc.edu/.

Data-Collection Objectives: Monitor the moisture content and moisture accumulation in the walls, as well as evaluating R-value of improved walls (if possible). Evaluate the relative humidity and temperatures within the wall to determine if conditions are right for mold growth.

Time Recording Standard: Always Alaska Standard Time (UTC – 9).

Datalogger Scan Interval Standard:  5 seconds

Time Measurement Standards:

- Hourly readings are recorded at the end of the hour; therefore, the hourly average air temperature, for example, with a 5-second scan interval and a time stamp of 14:00 is measured from 13:00:05 to 14:00:00.  Instantaneous readings are taken at the time specified by the time stamp.

- A day begins at midnight (00:00:00) and ends at midnight (23:59:55).  All daily data are from the day prior to the date of the time stamp.  For example, if the time stamp reads 09/09/2007 00:00 or 09/09/2007 12:00:00 AM, the data are from 09/08/2007.

Data Downloading: Daily at 2am, saved to http://cchrc.arsc.edu/

Wall Numbering Protocol:  Wall 1 is on the south east corner.  Numbering continues around until wall 5 is on the north wall and wall 9 is on the south west corner.

There will be 5 types of sensors used in the MTL. They will vary by wall. The 5 types are presented in Table A1.

| Relative Humidity | Temperature | Moisture Content | Pressure Transducer | Heat Flux Transducers |
| --- | --- | --- | --- | --- |
| Honeywell HIH4010 | US Sensor PS103J2 NTC Thermistors | Resistance pins | Setra Pressure transducer 265 | REBS HFT3, Huskeflux HFP01 |
| -40°C to 85°C 0% to 100% | -80°C to 150°C | 7% to 25%* | 0 to 249 Pa | -40°C to 55°C, ±100 W/m2 -30°C to +70°C, ±2000 W/m2 |
| ±3.5% | ±0.1°C | ±2% * | ±1% | ±5% -15% to +5% |
| VoltSe (MTLSI_raw,1,mV5000,2,1,0,250,1.0,0) 'Convert Raw voltage to RH MTLSI_RH=(MTLSI_raw-.8)/0.031 | Const a = 0.0011253 Const b = 0.00023131427 Const c = 0.000000109342 For i=1 To 26 Therm_kOhm(i) = (10*therm(i))/(1-therm(i)) [with 10 kohm resistor in bridge] Next i For i=1 To 26 D(i) = LN (1000*Therm_kOhm(i)) 'ln resistance (ohm) Next i For i=1 To 26 Temp_C(i) = (1/(a + b*D(i) + c*(D(i))^3)) - 273.15 'Steinhart & Hart Equation Next i | MoistureR_OHM(i)=103*(1-X(i))X(i) Log10(Moisturepercent) = 2.99 - 2.113 (log10(log10(MoistureR_OHM))) Corrected Moisture Percent =Moisturepercent(i)+0.567-(0.026*TempMC_C(i))+(0.000051*TempMC_C(i)2)(0.881*(1.0056TempMC_C(i))-BA A=0.813 B=1.888 for Coastal Douglas fir (Evans 2001) | VoltSe (setra_volt(1),1,mv5000,14,True,0,_60Hz,.001,0) pressure_pa(1)=(setra_volt(1))*12.45444 | VoltDiff (HeatFlux_mV,7,mV25C,1,True ,200,250,1.0,0) HeatFlux(1)= HeatFlux_mV(1) * multiplier |
| Overall MTL RH | Overall MTL Temp |  | Overall MTL pressure wrt outside |  |
| RH across the wall cavity | Temp across wall cavity | MC of studs and base plate in cavity |  | HF on interior face of wall |
| RH outside of sheathing | Temp outside of sheathing |  |  |  |

*These values are for Douglas fir, there is less accuracy for plywood. (Straube, Onysko, Schumacher, 2002)